# UMAT Implementation Documentation

# Modified Structural Constitutive Model (MSCM) v1.0

Babak Bozorgmehri and Michael S. Sacks
Willerson Center for Cardiovascular Modeling and Simulation
December 2024

# Table of Contents

1. Overview
2. Material Model Description
3. Implementation Details
4. Usage Instructions
5. Input Parameters
6. Output Variables
7. Compilation Instructions
8. Validation and Testing

# 1. Overview

This document describes the user-defined material (UMAT) implementation of the Modified Structural Constitutive Model (MSCM) v1.0 for LS-DYNA. The MSCM is an advanced hyperelastic material model designed specifically for soft biological tissues, such as heart valves, skin and blood vessels, particularly cardiovascular tissues that exhibit complex fiber-reinforced behavior. he MSCM focuses on detailed descriptions of collagen fiber kinematics to describe the mechanical responses, rather than basic phenomenology.

## Key Features:

• Fiber dispersion and reorientation tracking
• Gradual fiber recruitment based on stretch
• Plane stress formulation for membrane elements
• Real-time computation of fiber ensemble recruitment (FEFR)
• Comprehensive history variable output for post-processing

## Supported Elements:

• Solid elements (8-node hexahedral)
• Isogemetric Analysis (under development)

# 2. Material Model Description

## 2.1 Constitutive Framework

The MSCM decomposes the total second Piola-Kirchhoff stress into matrix and fiber contributions:

S = Sm + Sf

where:
• Sm = Matrix stress (Neo-Hookean) includes Isochoric (Siso) and volumetric (Svol) parts
• Sf = Fiber ensemble stress

## 2.2 Matrix Response

The matrix follows a compressible Neo-Hookean model with plane stress correction:

Sm = c̄m(I - C33C-1) + dpen ln(J) C-1

## 2.3 Fiber Response

The fiber ensemble incorporates:
• Angular distribution: Gaussian distribution with standard deviation σ_theta
• Recruitment distribution: Gaussian distribution with mean μs and standard deviation σ_s
• Linear fiber stress law: P = (γ̄f/λs)(λf/λs - 1)

# 3. Implementation Details

## 3.1 Integration Scheme

The fiber ensemble stress is computed using numerical integration:
• Theta quadrature: 21-point Gauss-Legendre over entire theta range
• Recruitment quadrature: 5-point Gauss-Legendre over [λlb, λf]
• Total integration points: 105 points (1×21×5)

## 3.2 Plane Stress Implementation

1. Solve for C33 = 1/C-133
2. Apply correction to matrix stress
3. No correction needed for fiber stress (in-plane fibers)

## 3.3 Subroutine Structure

Main computational flow:
1. Extract deformation gradient F from history variables
2. Compute right Cauchy-Green tensor C = F^TF
3. Calculate fiber stretches for all directions
4. Integrate fiber ensemble stress
5. Compute matrix stress with plane stress correction
6. Push-forward to Cauchy stress: σ = (1/J)FSF^T
7. Store results in history variables

# 4. Usage Instructions

## 4.1 LS-DYNA Keyword Input

Use *MAT_USER_DEFINED_MATERIAL_MODELS (MAT_047) with the following setup:

*MAT_USER_DEFINED_MATERIAL_MODELS

$#     mid        ro        mt       lmc       nhv    iortho      ibulk         ig

1   1.41E-6        47         9       145         1         2         1

$#   ivect     ifail    itherm    ihyper      ieos      lmca    unused    unused

0         0         0        -1         0         0

$#    aopt      mafc        xp        yp        zp        a1        a2        a3

2       1.0       0.0       0.0       0.0       1.0       0.0       0.0

$#      v1        v2        v3        d1        d2        d3      beta     ievts

0.0       0.0       0.0       0.0       1.0       0.0       0.0         0

$#     mu         K gamma_fbar   sigma_n     mu_s   Sigma_s       λ_lb      λ_ub

100     0.10e6    22966      33.0    1.118      0.04       1.01      1.24

$#	   nhv

145

## 4.2 Material Constants Array (CM)

| Parameter | Description | Units |
| --- | --- | --- |
| CM(1) | Matrix shear modulus (2×c₁₀) | kPa |
| CM(2) | Bulk modulus (2×d_pen) | kPa |
| CM(3) | Fiber modulus (γ̄_f) | kPa |
| CM(4) | Not used (set to 0) | - |
| CM(5) | Angular dispersion (σ_theta) | degrees |
| CM(6) | Recruitment mean (μ_s) | - |
| CM(7) | Recruitment std dev (σ_s) | - |
| CM(8) | Lower bound (λ_lb) | - |

# 5. Input Parameters

## 5.1 Typical Parameter Ranges

Based on cardiovascular tissue characterization:

| Parameter | Typical Range | Example Value |
| --- | --- | --- |
| Matrix modulus | 50-100 kPa | 100 kPa |
| Bulk modulus | 10-100 MPa | 100 MPa |
| Fiber modulus | 1000-5000 kPa | 2300 kPa |
| Angular dispersion | 10-50° | 33° |
| Recruitment mean | 1.05-1.20 | 1.118 |
| Recruitment std | 0.02-0.10 | 0.04 |
| Lower bound | 1.00-1.05 | 1.01 |
| Upper bound | 1.20-1.50 | 1.24 |

# 6. Output Variables

## 6.1 History Variables

The UMAT stores comprehensive data in history variables (HSV) for post-processing:

| HSV Range | Content | Description |
| --- | --- | --- |
| 1-9 | Deformation gradient F | 3×3 tensor components |
| 10-15 | Total stress S | Upper triangular components |
| 16-18 | Fiber stress Sf | S11, S22, S12 components |
| 19-21 | Matrix stress Sm | S11, S22, S12 components |
| 31-50 | Fiber angles β | Deformed angles in degrees |
| 51-70 | Gamma_t values | Transformed ODF |
| 71-90 | FEFR values | Fiber ensemble recruitment |
| 91-110 | Gamma_theta values | Reference ODF |
| 115 | Total recruitment TFR | Percentage (0-100%) |
| 141-143 | Principal stretches | λ_x, λ_y, max(λ_f) |

## 6.2 Post-Processing Outputs

Key quantities available for visualization:
• Fiber orientation distribution (ODF) evolution
• Fiber recruitment patterns (FEFR)
• Stress decomposition (matrix vs fiber contributions)
• Total fiber recruitment (TFR) percentage

# 7. Compilation Instructions

## 7.1 Environment Setup

Required software:
• Intel Fortran Compiler (ifort/ifx)
• LS-DYNA object files
• Intel MKL libraries

# 8. Validation and Testing

## 8.1 Validation Results

The MSCM UMAT has been validated against:
• Analytical solution: < 0.01% error in stress components
• Integration convergence: < 0.002 kPa error with 1×15×5 scheme

## 8.2 Test Cases

Recommended validation tests:
1. Equi-Biaxial tension: Check anisotropy
2. Simple shear: Test fiber reorientation
3.Inflation: Validate plane stress implementation

# 9. Known Issues and Limitations

Current limitations:

• Element support: Currently optimized for 8-node solid elements
• History variables: Limited to 145 HSV slots
• Implicit analysis: Material tangent not yet implemented

## Future Developments:

• Implementation of material tangent for implicit analysis
• GPU acceleration for large-scale simulations

# Contact Information

For questions, bug reports, or collaboration inquiries:

Babak Bozorgmehri
Email: babak.bozorgmehri@lut.fi
(Willerson Center for Cardiovascular Modeling and Simulation)

(UT Austin, USA)
LUT University, Finland

Michael S. Sacks
Email: msacks@oden.utexas.edu
Willerson Center for Cardiovascular Modeling and Simulation
UT Austin, USA